# Arcs Simulator — defect log

12–13 October 2026. The Python under Scripts was read, then the UI was run against an Aubo i5 at 192.168.2.124. Open, Bake, Simulate, Move, Upload, Run, and Stop were used. The code was not changed to fix these items. Twenty-six defects are listed below. Numbers are the original note numbers, so a gap means an earlier note was checked and is not a defect.

## What was checked, and what was not

Simulate does honor the repeat count, so the ghost can loop. The real arm does not, because of item 41. Pause and Halt do run: Pause toggles the ghost, and Halt calls StopMove and AbortPathBuffer. Holding Move is a joint move to one pose, which is what that button is for. At the end of Simulate the ghost returns to the pose saved when play started. That is written into exit_simulation_mode. A Sony camera was not connected, so item 11 was not seen live. Pause was not pressed while a real path was still running, so item 8 and item 14 were not seen on the arm.

## Speed check and upload

### 1. The speed check lets a fast path through

path_bake.py — evaluate_timing

Each small hop can be over the joint limit, and that failure is then cleared. The only check that can fail a span is the average joint rate from one keyframe to the next, with a 10 percent margin. Acceleration is not used. A path that passes this check can still be uploaded. On a connected run a bake was marked all ok and then played at about 0.51 m/s.

### 2. The first fast span paints every later span yellow

path_bake.py — evaluate_timing and the timeline color map

evaluate_timing never sets its blocked flag, so later spans are still scored. The color map has its own flag: the first span marked too fast forces every later span to uncalc. Uncalc is the yellow state, which is supposed to mean the speed has not been checked yet.

### 4. Ease timing ignores distance along the path

path_bake.py — time profile

Ease and custom Bezier modes place sample times by the curve parameter, not by distance along the path. A segment can be fast in the middle and still pass the speed check.

### 6, 20, and 26. The overspeed file is written where the download cannot see it

path_bake.py — write_overspeed_report; webui.py — /api/speed_export

Bake writes \home\workdir\artifacts\bake_overspeed.csv. The download route looks for Scripts/bake_overspeed.csv. There is no export_overspeed function. The same Linux path was written on every bake in these runs, including bakes with 0 rows and the 100-waypoint bake with 47 rows.

### 33. An all-ok bake can still produce overspeed rows

path_bake.py — write_overspeed_report

A 99-span bake finished with every span marked ok, and the overspeed file still contained 47 rows. This is the same check as item 1, seen on a long path.

### 40. The first path-buffer check is rejected, then the retry is treated as success

arcscontrol.py — pathBufferEval

Uploaded points are resampled to one every 0.01 s (71 points became 2401, 29 points became 525). The first pathBufferEval is called with empty limit arrays and returns -5, which this code treats as invalid arguments. A second call that passes joint limits returns 0, and the upload is reported ok. Seen twice on the connected arm.

## Files, timeline, and the viewer

### 3. Snap to grid is saved and never applied

waypoints.py — add_waypoint; webui.py settings

snap_to_grid is stored in settings and sent back by the status API. add_waypoint does not snap the new point. Importing a raw JSON file also writes the setting back to false.

### 18 and 23. Opening a file loads the UI program a second time

file_manage.py — apply_loaded_doc

The running app is the main module. Opening a file does import webui to read settings, which loads Scripts/webui.py again and runs its startup code. debug_trace.reset() restarts the debug clock and truncates the debug log in the middle of Open. Seen on the connected runs as well as the earlier ones.

### 19 and 24. One slow request stalls the clock and the camera poll

webui.py Flask process; index.html polls

Flask serves one request at a time. While Bake or a burst of debug posts holds that thread, /api/camera_state and /api/clock wait in the queue. The speed graph then jumps in time while the speed value itself has not spiked. The multi-second waits in the longest log were made worse by temporary debug logging. The single-threaded queue is still there.

### 21. The speed graph axis never comes back down

index.html — pushSpeedSample

The vertical scale is the largest sample seen so far. After one fast reading, later slower motion is drawn flat against that peak.

### 28. A finished bake is re-scored many times a second

webui.py — /api/status

After a bake, is_runnable is called about eight times a second and padded_timed_joints about once a second, each returning the same samples. Each call is cheap. Together with the camera poll they keep the request queue busy.

### 34. A large Open is slow because every waypoint is applied on that request

file_manage.py — apply_loaded_doc

Opening the 100-waypoint file took about 2.2 s on the server after the per-waypoint debug lines were removed, and about 3.7 s while those lines were still posted. tick_cameras itself was under a millisecond. The wait is the Open work, plus item 18 loading the UI module again.

## Arm connection and motion

### 5 and 41. Repeat never reaches the arm, so a loop cannot start

webui.py — /api/real/upload; robot_runtime.py — tick_real_path

The repeat control sends a field named repeat. Simulate reads that field, so the ghost can loop. The real upload only reads a field named loops, so the arm stores 0. A path uploaded with repeat 1 ran once, finished, and then called StopMove and AbortPathBuffer. The code that would start the buffer again without aborting it only runs when the stored loop count is greater than 0, so that path has not been reached.

### 7. A powered-off controller can be copied as a real pose

robot_runtime.py — robot_connected

Any six joint numbers count as connected, including all zeros. While the controller is in PowerOff those zeros are copied into the viewer and the on-screen arm jumps there. With the console on, the connected arm reported real joint values and this did not happen.

### 8. Pause does not abort a path-buffer move

robot_runtime.py — pause_real_path

Stop calls AbortPathBuffer, which frees the buffer. Pause, when a real path is actually running, only sets the speed fraction to 0 and calls stopMove. The code comment on abort says stopMove alone does not abort movePathBuffer. The pause tests in this log were pressed after the path had already stopped, so this case has not been seen on the arm.

### 10. A valid inverse-kinematics answer can be discarded

arcscontrol.py — GetIK

After the controller returns six joint angles, GetIK rejects them when the first three numbers are close to the target x, y, and z. Those are radians and meters. A real solution can be dropped because the numbers happen to match. Live GetIK during a drag returned normal joint angles and did not hit this check.

### 14. Resume can cut the path with the wrong clock

robot_runtime.py — pause_real_path

If the buffer is rebuilt after a pause, samples are kept when their time is at or after the playback clock. That clock includes camera preroll. Sample times start at zero, so the tail can start at the wrong pose. Resume of a running real path was not tested. Pause was pressed only after Stop.

### 27. A closed robot port still blocks the viewer for about 0.35 s

arcscontrol.py — connect

When 192.168.2.124 port 30004 is closed, each reconnect takes about 0.35 s on the same thread as the viewer. The old 21 s SDK connect is avoided. A successful login, once the controller was up, took 0.05 s.

### 31. Backup IK keeps one stale seed for the whole bake

robot_runtime.py — solve_backup_ik

On a 100-waypoint bake with the arm disconnected, almost every attempt used the same live joints. That seed failed, home was tried, and the same target was solved again, about 0.1 s each time. The bake of 99 spans took 13.8 s. When the seed was the previous waypoint, the position error dropped to about 1 mm.

### 32. A home IK result can be kept with a large orientation error

robot_runtime.py — backup IK home seed

A home solution is rejected only when the position error is over 0.04 m. Orientation is not part of that cutoff. One home result with a position error of 0.020 m and an orientation error of 0.37 rad was kept.

## Camera

### 11. Sony recording runs on the empty frame

waypoints.py — Sony camera tick

When grab_jpeg returns no frame, the record path still runs and tries to decode that empty buffer. A successful JPEG updates the preview and does not reach the PC-side recorder. No camera was connected in these tests, so this was not seen live.